NOTE: NEGATIVE OPD IS DIVERGING, POSITIVE OPD IS CONVERGING

C     SYSTEM() --    This is the array which keeps track of lens database

C                  parameters which as not surface dependent.

C                  There are 130 entries in this array for every lens.

C                  The array contains:

C     SYSTEM(1)  -- Wavelength in microns for wavelength #1

C     SYSTEM(2)  -- Wavelength in microns for wavelength #2

C     SYSTEM(3)  -- Wavelength in microns for wavelength #3

C     SYSTEM(4)  -- Wavelength in microns for wavelength #4

C     SYSTEM(5)  -- Wavelength in microns for wavelength #5

C     SYSTEM(6)  -- Units identifier 1=inches

C                                    2=cm

C                                    3=mm

C                                    4=meters

C     SYSTEM(7)  -- First wavelength of the primary wavelength pair.

C     SYSTEM(8)  -- Second wavelength of the primary wavelength pair.

C     SYSTEM(9)  -- First wavelength of the secondary wavelength pair.

C     SYSTEM(10) -- Second wavelength of the secondary wavelength pair.

C     SYSTEM(11) -- Control wavelength number (reference wavelength).

C     SYSTEM(12) -- SAY value

C     SYSTEM(13) -- SAX value

C     SYSTEM(14) -- SCY value

C     SYSTEM(15) -- Y1 value for the SCY value

C     SYSTEM(16) -- SCX value

C     SYSTEM(17) -- X1 value for the SCX value

C     SYSTEM(18) -- Flag for SCY FANG explicitly set(1.0), else = 0.0

C     SYSTEM(19) -- Flag for SCX FANG explicitly set(1.0), else = 0.0

C     SYSTEM(20) -- Surface number of the last surface.

C     SYSTEM(21) -- SCY FANG value.

C     SYSTEM(22) -- Y1 value for SCY FANG

C     SYSTEM(23) -- SCX FANG value.

C     SYSTEM(24) -- X1 value for SCY FANG

C     SYSTEM(25) -- Surface number of the REFERENCE surface.(default=1)

C     SYSTEM(26) -- Surface number of the ASTOP surface. (-99 if no ASTOP)

C     SYSTEM(27) -- ASTOP adjustment flag

C                      1.0=EN

C                     -1.0=EX

C                      0.0=(none)

C                      2.0=ENEX

C     SYSTEM(28) -- XZ bilateral symmetry flag (1=yes,0=no)

C     SYSTEM(29) -- Flag for generalized paraxial ray trace (1=on,0=off)

C     SYSTEM(30) -- Mode flag

C                      1.0=FOCAL

C                      2.0=UFOCAL

C                      3.0=AFOCAL

C                      4.0=UAFOCAL

C     SYSTEM(31) -- Spectral weighting factor for wavelength #1

C     SYSTEM(32) -- Spectral weighting factor for wavelength #2

C     SYSTEM(33) -- Spectral weighting factor for wavelength #3

C     SYSTEM(34) -- Spectral weighting factor for wavelength #4

C     SYSTEM(35) -- Spectral weighting factor for wavelength #5

C     SYSTEM(36) -- YZ transverse MAG reset value

C     SYSTEM(37) -- XZ transverse MAG reset value

C     SYSTEM(38) -- YZ desired magnification

C     SYSTEM(39) -- XZ desired magnification

C     SYSTEM(40) -- Beginning surface YZ mag)

C     SYSTEM(41) -- Beginning surface XZ mag)

C     SYSTEM(42) -- Ending surface YZ mag)

C     SYSTEM(43) -- Ending surface XZ mag)

C     SYSTEM(44) -- F/# or F/# hold flag (1 or -1) YZ

C     SYSTEM(45) -- F/# or F/# hold flag (1 or -1) XZ

C     SYSTEM(44) -- Expup or Expup hold flag (2 or -2) YZ

C     SYSTEM(45) -- Expup or Expup hold flag (2 or -2) xZ

C     SYSTEM(46) -- F/# hold value YZ

C     SYSTEM(47) -- F/# hold value XZ

C     SYSTEM(48) -- YZ bilateral symmetry flag (1=yes,0=no)

C     SYSTEM(49) -- if 0, XZ data not entered for field or aperture

C     SYSTEM(49) -- if 1, XZ data entered for aperture only

C     SYSTEM(49) -- if 2, XZ data entered for field only

C     SYSTEM(49) -- if 3, XZ data entered for field and aperture

C     SYSTEM(50) -- flag tracks current configuration (don't mess with this!)

C     SYSTEM(51) -- = 1.0 if Y1(SCY) was explicitly set, else is = 0.0

C     SYSTEM(52) -- = 1.0 if X1(SCX) was explicitly set, else is = 0.0

C     SYSTEM(53) -- = 1.0 if Y1(SCY FANG) was explicitly set, else is = 0.0

C     SYSTEM(54) -- = 1.0 if X1(SCX FANG) was explicitly set, else is = 0.0

C     SYSTEM(55) -- = used by subroutine STH.FOR (don't mess with this!)

C     SYSTEM(56) -- = number of the highest number non-blank alternate config.

C     SYSTEM(57) -- = NOT IN USE

C     SYSTEM(58) -- = 0 means generalized paraxial trace "off", 1 means "on"

C     SYSTEM(59) -- = reference surface orientation angle

C     SYSTEM(60) -- = 0 if SCY or SCY FANG not explicitly set

C     SYSTEM(60) -- = 1 if SCY or SCY FANG explicitly set

C     SYSTEM(61) -- = 0 if SCX or SCX FANG not explicitly set

C     SYSTEM(61) -- = 1 if SCX or SCX FANG explicitly set

C     SYSTEM(62) -- = 1, REGULAR RAY AIMING IS ON, 0= AIMING OFF

C     SYSTEM(63) -- = 1, TELECENTRIC RAY AIMING IS ON, 0= TELE AIMING OFF

C     SYSTEM(64) -- = 1, NAOY EXPLICITLY SET, 2= NAOX EXPLICITLY SET ,3= BOTH SET

C     SYSTEM(65) -- = NAOY value

C     SYSTEM(66) -- = NAOX value

C     SYSTEM(67) -- = 1, FNOY EXPLICITLY SET, 2= FNOX EXPLICITLY SET ,3= BOTH SET

C     SYSTEM(68) -- = FNOY value

C     SYSTEM(69) -- = FNOX value

C     SYSTEM(70) -- = 0.0 (no aplanatic ray aiming, 1.0 (aplanatic ray aiming)

C     SYSTEM(71) -- Wavelength in microns for wavelength #6

C     SYSTEM(72) -- Wavelength in microns for wavelength #7

C     SYSTEM(73) -- Wavelength in microns for wavelength #8

C     SYSTEM(74) -- Wavelength in microns for wavelength #9

C     SYSTEM(75) -- Wavelength in microns for wavelength #10

C     SYSTEM(76) -- Spectral weighting factor for wavelength #6

C     SYSTEM(77) -- Spectral weighting factor for wavelength #7

C     SYSTEM(78) -- Spectral weighting factor for wavelength #8

C     SYSTEM(79) -- Spectral weighting factor for wavelength #9

C     SYSTEM(80) -- Spectral weighting factor for wavelength #10

C     SYSTEM(81) -- is AIMOFFSET in X

C     SYSTEM(82) -- is AIMOFFSET in Y

C     SYSTEM(83) -- =1 means SAY floats, =0 means SAY does not float

C     SYSTEM(84) -- =1 means SAX floats, =0 means SAX does not float

C     SYSTEM(85) -- WRX gaussian beam setting.

C     SYSTEM(86) -- WRY gaussian beam setting.

C     SYSTEM(87) -- BDX gaussian beam setting.

C     SYSTEM(88) -- BDY gaussian beam setting.

C     SYSTEM(89) -- is AIMOFFSET in Z

C     SYSTEM(90) -- LENS TILT RET resolution flag

C                   0=NOT RESOLVED

C                   1=RESOLVED

C     SYSTEM(91) -- AUTOFUNC SETTING 0 TO 10, 0=NO CALL, 1-10= FUNC# TO CALL

C     SYSTEM(92) -- PXIM OR PXIM FANG value

C     SYSTEM(93) -- PYIM OR PYIM FANG value

C     SYSTEM(94) -- Flag for PXIM FANG explicitly set(1.0), else = -1.0

C                   if PXIM else 0.0

C     SYSTEM(95) -- Flag for PYIM FANG explicitly set(1.0), else = -1.0

C                   if PYIM else 0.0

C     SYSTEM(96) -- RXIM OR RXIM FANG value

C     SYSTEM(97) -- RYIM OR RYIM FANG value

C     SYSTEM(98) -- Flag for RXIM FANG explicitly set(1.0), else = -1.0

C                   if RXIM else 0.0

C     SYSTEM(99) -- Flag for RYIM FANG explicitly set(1.0), else = -1.0

C                   if RYIM else 0.0

C     SYSTEM(100)-- 0 FOR NO REVERSE TRACE, 1 FOR REVERSE TRACE

C     SYSTEM(101)-- 0 FOR NON-VERBOSE OPTIMIZATION OUTPUT

C                   1 FOR VERBOSE (CONTROLLED VIA THE "OVERBOSE" COMMAND

C     SYSTEM(102)-- 1 IF NSS SURFACES ARE PRESENT, 0 IF NO NSS SURFACES ARE PRESENT

C     SYSTEM(103)-- 0 FOR NO SCREEN SURFACE, 1 FOR A SCREEN SURFACE

C     SYSTEM(104)-- SURFACE NUMBER OF THE SCREEN SURFACE

C     SYSTEM(105)-- DIAMETER OF HOLES IN SCREEN SURFACE

C     SYSTEM(106)-- THICKNESS OF SCREEN SURFACE (DEPTH OF THE HOLES)

C     SYSTEM(107)-- X AND Y SPACING, ON A RECTANGULAR GRID BETWEEN HOLES

C     SYSTEM(108)-- NOT IN USE

C     SYSTEM(109)-- NOT IN USE

C     SYSTEM(110)-- NOT IN USE

C     SYSTEM(111)-- WV1 IN MICRONS

C     SYSTEM(112)-- WV2 IN MICRONS

C     SYSTEM(113)-- WV3 IN MICRONS

C     SYSTEM(114)-- WV4 IN MICRONS

C     SYSTEM(115)-- WV5 IN MICRONS

C     SYSTEM(116)-- WV6 IN MICRONS

C     SYSTEM(117)-- WV7 IN MICRONS

C     SYSTEM(118)-- WV8 IN MICRONS

C     SYSTEM(119)-- WV9 IN MICRONS

C     SYSTEM(120)-- WV10 IN MICRONS

C     SYSTEM(121) -- NUMBER OF PIXELS IN THE X DIMENSION OF THE SPOT DIAGRAM SURFACE

C     SYSTEM(122) -- NUMBER OF PIXELS IN THE Y DIMENSION OF THE SPOT DIAGRAM SURFACE

C     SYSTEM(123) -- PIXEL PITCH IN THE X DIMENSION OF THE SPOT DIAGRAM SURFACE

C     SYSTEM(124) -- PIXEL PITCH IN THE Y DIMENSION OF THE SPOT DIAGRAM SURFACE

C     SYSTEM(124) -- X COORDINATE OF THE CENTER OF THE DETECTOR ARRAY ON THE SPOT DIAGRAM SURFACE

C     SYSTEM(126) -- Y COORDINATE OF THE CENTER OF THE DETECTOR ARRAY ON THE SPOT DIAGRAM SURFACE

C     SYSTEM(127) -- SURFACE NUMBER OF THE SPOT DIAGRAM SURFACE (EXPLICITLY ASSIGNED IN NSS BY IS THE

C                      CURRENT OBJECT SURFACE IN SEQUENTIAL SPOT DIAGRAMS.

C     SYSTEM(128) -- 0 FOR FLIPREFX OFF, 1 FOR ON

C     SYSTEM(129) -- 0 FOR FLIPREFY OFF, 1 FOR ON

C

C     ARRAAYS PFLDS(1:2,1:10) AND CFLDS(1:2,1:10) STORE THE X AND Y

C     VALUES OF THE MULTIPLE FIELDS OF VIEW DEFINED WITH A LENS

C     FLDCNT COUNTS THE NUMBER OF CURRENT ACTIVE FIELDS.

C

C     FTFL01(J,SURF#) holds all 96 special surface coefficients

C     which the user can both input and also vary during optimization

C     J ranges from 1 to 96. All 96 coefficients are available in the

C     type 21 user-defined surface

C

C     The ALENS(J,SURF#) array holds all the surface dependent lens

C     database information.

C     ALENS(1,surf#) -- The surface curvature.

C     ALENS(2,surf#) -- The surface conic constant.

C     ALENS(3,surf#) -- The surface thickness to the next surface

C     ALENS(4,surf#) -- 4th order aspheric coefficient

C     ALENS(5,surf#) -- 6th order aspheric coefficient

C     ALENS(6,surf#) -- 8th order aspheric coefficient

C     ALENS(7,surf#) -- 10th order aspheric coefficient

C     ALENS(8,surf#) -- asphere flag, 1= asphere, 0 = no asphere

C     ALENS(9,surf#) -- Clear aperture type flag

C                       0.0=NONE

C                       1.0=CIRCULAR

C       ALENS(10,SURF#)=CLAP RADIUS

C       ALENS(11,SURF#)=RAD TO FLT

C       ALENS(12,SURF#)=YD

C       ALENS(13,SURF#)=XD

C       ALENS(14,SURF#)=ZD

C                       2.0=RECTANGULAR

C       ALENS(10,SURF#)=Y

C       ALENS(11,SURF#)=X

C       ALENS(12,SURF#)=YD

C       ALENS(13,SURF#)=XD

C                       3.0=ELLIPTICAL

C       ALENS(10,SURF#)=Y

C       ALENS(11,SURF#)=X

C       ALENS(12,SURF#)=YD

C       ALENS(13,SURF#)=XD

C                       4.0=RACETRACK

C       ALENS(10,SURF#)=Y

C       ALENS(11,SURF#)=X

C       ALENS(12,SURF#)=YD

C       ALENS(13,SURF#)=XD

C       ALENS(14,SURF#)=RAD OF CORNER

C                       5.0=REGULAR POLYGON

C       ALENS(10,SURF#)=RAD TO CORNER

C       ALENS(11,SURF#)=N (NUMBER OF SIDES)

C       ALENS(12,SURF#)=YD

C       ALENS(13,SURF#)=XD

C                       6.0=IRREGULAR POLYGON

C       ALENS(10,SURF#)=POLYGON DEFINITION FILE # (1 TO 99) IPOLY01.DAT TO IPOLY99.DAT

C       ALENS(11,SURF#)=NUMBER OF POINTS

C       ALENS(12,SURF#)=YD

C       ALENS(13,SURF#)=XD

C       ALENS(14,SURF#)=MAXIMUM DIMENSION FOR SYS12/SYS13 SETTINGS

C     ALENS(15,surf#)=CLAP TILT

C     ALENS(16,surf#) -- Obscuration type flag

C                       0.0=NONE

C                       1.0=CIRCULAR

C                       2.0=RECTANGULAR

C                       3.0=ELLIPTICAL

C                       4.0=RACETRACK

C                       5.0=REGULAR POLYGON

C                       6.0=IRREGULAR POLYGON

C     ALENS(17,surf#) TO ALENS(22,surf#)

C     ALENS(23,surf#) -- toric surface flag

C                       0.0=not toric

C                       1.0=Y-toric

C                       2.0=X-toric

C     ALENS(24,surf#) -- toric curvature value

C     ALENS(25,surf#) -- Tilt flag

C                       0.0=NO TILT (DEFAULT VALUE)

C                       1.0= TILT

C                      -1.0= RTILT

C                       2.0= TILT AUTO

C                       3.0= TILT AUTOM

C                       4.0= TILT BEN

C                       5.0= TILT DAR

C                       6.0= TILT RET

C                       7.0= TILT REV

C     ALENS(26,SURF) TO ALENS(28,SURF) STORE THE ALPHA,BETA AND GAMMA

C                       ANGLES IN DEGREES

C     ALENS(29,surf#) -- Decenter flag, 0= no decenter, 1= decenter

C     ALENS(30,surf#) -- Y-decenter value

C     ALENS(31,surf#) -- X-decenter value

C     ALENS(69,surf#) -- Z-decenter value

C     ALENS(32,surf#) -- is the pickup flag

C                       0.0=NO PIKUPS

C                       GREATER THAN 0.0 = NUMBER OF PIKUPS ON A SURFACE

C     ALENS(33,surf#) -- IS THE SOLVES FLAG

C                       0.0=NO SOLVES

C                       1.0=YZ THICKNES SOLVES

C                       2.0=YZ CURVATURE SOLVES

C                       3.0=YZ CURVATURE AND THICKNESS SOLVES

C                       0.1=XZ THICKNES SOLVES

C                       0.2=XZ CURVATURE SOLVES

C                       0.3=XZ CURVATURE AND THICKNESS SOLVES

C     IF ALL SOLVES PRESENT, ALENS(33,SURF) CAN GET TO 3.3

C     ALENS(34,surf#) -- is the special surface flag = to the special

C                        surface type. If negative, the special surface

C                        is currently shut "OFF".

C     ALENS(35,surf#) -- ASI flag, 0 = no ASI, 1= ASI (alternate surf int)

C     ALENS(36,surf#) -- anamorphic asphere, 0 = none, 1 = anamorphic

C     ALENS(37,surf#) -- 4th order anamorphic aspheric term

C     ALENS(38,surf#) -- 6th order anamorphic aspheric term

C     ALENS(39,surf#) -- 8th order anamorphic aspheric term

C     ALENS(40,surf#) -- 10th order anamorphic aspheric term

C     ALENS(41,surf#) -- anamorphic conic coefficient

C     ALENS(42,surf#) -- config data flag, 0 = none, 1 = config data exists

C     ALENS(43,surf#) -- 2nd order aspheric coefficient (plano surfaces only)

C     ALENS(44,surf#) -- surface label flag, 0 = no label, 1 = label exists

C     ALENS(45,surf#) -- NOT USED

C     ALENS(46,surf#) -- refractive index at wavelength #1

C     ALENS(47,surf#) -- refractive index at wavelength #2

C     ALENS(48,surf#) -- refractive index at wavelength #3

C     ALENS(49,surf#) -- refractive index at wavelength #4

C     ALENS(50,surf#) -- refractive index at wavelength #5

C     ALENS(69,surf#) -- Z-decenter value

C     ALENS(51,surf#) -- Clear aperture erase type flag

C                       0.0=NONE

C                       1.0=CIRCULAR

C                       2.0=RECTANGULAR

C                       3.0=ELLIPTICAL

C                       4.0=RACETRACK

C                       5.0=REGULAR POLYGON

C                       6.0=IRREGULAR POLYGON

C     ALENS(52,surf#) TO ALENS(57,surf#)

C     ALENS(58,surf#) -- 0.0 OR 1.0, IF 1.0, FORCES RAYS TO BE BLOCKED

C                        BY CLAPS AND COBS IN FOOTPRINT TRACING IN QTRA.FOR

C                        DEFAULT IS 0.0

C       ON 10/28/2004    ADDED THE OPTION IN "RAY CAOB" THAT IF FOOTBLOCK WAS ON OR SET TO 1

C       NORMAL APERTURE CHECKING WOULD BE SKIPPED. A DOUBLE POSITIVE MAKES A NEGATIVE.

C     ALENS(59,surf#) -- PIVOT FLAG (0= at origin, 1= alternate pivot defined

C                        in ALENS 78,79 and 80 (X,Y,Z fom surface vertex)

C     ALENS(60,surf#) -- (not in use)

C     ALENS(61,surf#) -- Obscuration erase type flag

C                       0.0=NONE

C                       1.0=CIRCULAR

C                       2.0=RECTANGULAR

C                       3.0=ELLIPTICAL

C                       4.0=RACETRACK

C                       5.0=REGULAR POLYGON

C                       6.0=IRREGULAR POLYGON

C     ALENS(62,surf#) TO ALENS(67,surf#)

C     hold oscuration data (don't mess with these!).

C     ALENS(68,surf#) -- DUMMY SURFACE FLAG, 0 = DUMMY, 1= NOT DUMMY

C     ALENS(69,surf#) -- Z-decenter value

C     ALENS(70,surf#) -- designates the RET surf# for TILT RET

C     ALENS(71,surf#) -- refractive index at wavelength #6

C     ALENS(72,surf#) -- refractive index at wavelength #7

C     ALENS(73,surf#) -- refractive index at wavelength #8

C     ALENS(74,surf#) -- refractive index at wavelength #9

C     ALENS(75,surf#) -- refractive index at wavelength #10

C     ALENS(76,surf#) -- current "INR" surface value

C     ALENS(77,surf#) -- TILT RETURN RESOLUTION FLAG, SURFACE

C                        0=not resolved

C                        1=resolved

C     ALENS(78,surf#) -- TILT PIVOT X-LOCATION

C     ALENS(79,surf#) -- TILT PIVOT Y-LOCATION

C     ALENS(80,surf#) -- TILT PIVOT Z-LOCATION

C     ALENS(81,surf#) -- 12th order aspheric coefficient

C     ALENS(82,surf#) -- 14th order aspheric coefficient

C     ALENS(83,surf#) -- 16th order aspheric coefficient

C     ALENS(84,surf#) -- 18th order aspheric coefficient

C     ALENS(85,surf#) -- 20th order aspheric coefficient

C     ALENS(86, ) IS THE REFRACTIVE INDEX INPUT VALUE FOR FICT GLASS

C     ALENS(87, ) IS THE ABBE-NUMBER INPUT VALUE FOR FICT GLASS

C     ALENS(88, ) IS THE READ GRID DATA FLAG, 0 NOT YET READ, 1 READ ALREADY

C     ALENS(89, ) IS THE PARTIAL DISPERSION TO USE IN MODEL GLASS

C     ALENS(90, ) IS GLOBAL X-DECENTER

C     ALENS(91, ) IS GLOBAL Y-DECENTER

C     ALENS(92, ) IS GLOBAL Z-DECENTER

C     ALENS(93, ) IS GLOBAL ALPHA TILT

C     ALENS(94, ) IS GLOBAL BETA  TILT

C     ALENS(95, ) IS GLOBAL GAMMA TILT

C     ALENS(96, ) IS GRATING FLAG, 0.0=NO GRATING, 1.0= GRATING

C     ALENS(97, ) IS GRATING ORDER

C     ALENS(98, ) IS GRATING SPACING (LENS UNITS)

C     ALENS(99, ) IS GRATING X-DIR NUMBER

C     ALENS(100, ) IS GRATING Y-DIR NUMBER

C     ALENS(101, ) IS GRATING Z-DIR NUMBER

C     ALENS(102, ) SPECIFIC GRAVITY IN GRAMS/CC

C     ALENS(103, ) 0 OR 1, FLAG FOR DEFORMABLE SURFACE

C     ALENS(104, ) FILE ID FOR DEFORMABLE SURFACE

C     ALENS(105, ) TOTAL NUMBER OF ACTUATOR

C     ALENS(106, ) Z-SCALE FOR DEFORMABLE SURFACE

C     ALENS(107, ) (not in use)

C     ALENS(108, ) (not in use)

C     ALENS(109, ) DEFORMABLE SURFACE UPDATE FLAG (WORKS LIKE ALENS(88, )

C     ALENS(110, ) MIRROR THICKNESS

C     ALENS(111, ) COST PER UNIT WEIGHT

C     ALENS(112, ) COATING FILE NUMBER 1 TO 1000, 0 IS ALSO NO COATING

C     ALENS(113, ) 0 FOR PIVAXIS LOCAL, 1 FOR PIVAXIS NORMAL (LOCAL=DEFAULT)

C     ALENS(114, ) PERMANENT DX,TERM ADDED WITH A DX WITH A PIVOT

C     ALENS(115, ) PERMANENT DY,"                 DY            "

C     ALENS(116, ) PERMANENT DZ,"                 DZ            "

C     ALENS(117, ) ADDITIONAL DECENTER FLAG FOR PIVOTED SURFACES 0 FOR NONE,

C                  1 FOR SOME DECENTER TO ADD.

C     ALENS(118, ) PERMANENT ALPHA

C     ALENS(119, ) PERMANENT BETA

C     ALENS(120, ) PERMANENT GAMMA

C     ALENS(121, ) FOCAL LENGTH OF AN IDEAL LENS

C     ALENS(122, ) FIRST THREE DIGITS OF GLASS CODE

C     ALENS(123, ) SECOND THREE DIGITS OF GLASS CODE

C     ALENS(124, ) 0 FOR A REAL SURFACE, 1 FOR A PARAXIAL SURFACE (9/27/99)

C     ALENS(125, ) REFLECTION MODE

C                       0 = REFL

C                       1 = REFLTIRO

C                       2 = REFLTIR

C     ALENS(126, ) CCR/ROOF 0=NO,1=ROOF,2=CCR

C     ALENS(127, ) 0=NO MULTI-CLAP,  1.0=MULTI CLAP

C     ALENS(128, ) 0=NO MULTI-COBS,  1.0=MULTI COBS

C     ALENS(129, ) (not in use)

C     ALENS(130, ) NOT USED YET

C     ALENS(131, ) ARRAY LENS DX

C     ALENS(132, ) ARRAY LENS DY

C     ALENS(133, ) ARRAY LENS ODD=-1,EVEN=1,NONE=0

C     ALENS(134, ) 0=NO SPIDER, 1=SPIDER

C     ALENS(135, ) SPIDER N VALUE

C     ALENS(136, ) SPIDER W VALUE

C     ALENS(137, ) SPIDER L VALUE

C     ALENS(138, ) CCR OR ROOF NW1

C     ALENS(139, ) CCR OR ROOF NW2

C     ALENS(140, ) CCR OR ROOF NW3

C     ALENS(141, ) CCR OR ROOF NW4

C     ALENS(142, ) CCR OR ROOF NW5

C     ALENS(143, ) 0 IF INR IMPLICIT, ELSE 1 FOR INR EXPLICIT

C     ALENS(144, ) RANDOM RAY ERROR IN ARC SECONDS

C     ALENS(145, ) NOT USED YET

C     ALENS(146, ) NOT USED YET

C     ALENS(147, ) NOT USED YET

C     ALENS(148, ) NOT USED YET

C     ALENS(149, ) NOT USED YET

C     ALENS(150, ) NOT USED YET

C     ALENS(151, ) NOT USED YET

C     ALENS(152, ) NOT USED YET

C     ALENS(153, ) NOT USED YET

C     ALENS(154, ) NOT USED YET

C     ALENS(155, ) NOT USED YET

C     ALENS(156, ) NOT USED YET

C     ALENS(157, ) NOT USED YET

C     ALENS(158, ) NOT USED YET

C     ALENS(159, ) NOT USED YET

C     ALENS(160, ) NOT USED YET

C

C     MULTCLAP(1:1000,1:3,0:MAXSUR) 1=X, 2=Y, 3=GAMMA

C     MULTCOBS(1:1000,1:3,0:MAXSUR) 1=X, 2=Y, 3=GAMMA

C

C     TOL ARRAY KEEPS TRACK OF TOLERANCE VARIABLE VALUES

C     ADDED 2/25/98

C     TOLER(1, )  IS VALUE FOR DISPX

C     TOLER(2, )  IS SECOND SURFACE NUMBER FOR DISPX

C     TOLER(3, )  IS VALUE FOR DISPY

C     TOLER(4, )  IS SECOND SURFACE NUMBER FOR DISPY

C     TOLER(5, )  IS VALUE FOR DISPZ

C     TOLER(6, )  IS SECOND SURFACE NUMBER FOR DISPZ

C     TOLER(7, )  IS VALUE FOR STILTA

C     TOLER(8, )  IS X PIVOT FOR STILTA

C     TOLER(9, ) IS Y PIVOT FOR STILTA

C     TOLER(10, ) IS Z PIVOT FOR STILTA

C     TOLER(11, ) IS VALUE FOR STILTB

C     TOLER(12, ) IS X PIVOT FOR STILTB

C     TOLER(13, ) IS Y PIVOT FOR STILTB

C     TOLER(14, ) IS Z PIVOT FOR STILTB

C     TOLER(15, ) IS VALUE FOR STILTC

C     TOLER(16, ) IS X PIVOT FOR STILTC

C     TOLER(17, ) IS Y PIVOT FOR STILTC

C     TOLER(18, ) IS Z PIVOT FOR STILTC

C     TOLER(19, ) IS VALUE FOR BTILTA

C     TOLER(20, ) IS SECOND SURFACE NUMBER FOR BTILTA

C     TOLER(21, ) IS X PIVOT FOR BTILTA

C     TOLER(22, ) IS Y PIVOT FOR BTILTA

C     TOLER(23, ) IS Z PIVOT FOR BTILTA

C     TOLER(24, ) IS VALUE FOR BTILTB

C     TOLER(25, ) IS SECOND SURFACE NUMBER FOR BTILTB

C     TOLER(26, ) IS X PIVOT FOR BTILTB

C     TOLER(27, ) IS Y PIVOT FOR BTILTB

C     TOLER(28, ) IS Z PIVOT FOR BTILTB

C     TOLER(29, ) IS VALUE FOR BTILTC

C     TOLER(30, ) IS SECOND SURFACE NUMBER FOR BTILTC

C     TOLER(31, ) IS X PIVOT FOR BTILTC

C     TOLER(32, ) IS Y PIVOT FOR BTILTC

C     TOLER(33, ) IS Z PIVOT FOR BTILTC

C     TOLER(34, ) IS VALUE FOR ROLLX

C     TOLER(35, ) ENDING SURFACE NUMBER FOR ROLLX

C     TOLER(36, ) ALTERNATE SURFACE NUMBER FOR ROLLX

C     TOLER(37, ) IS VALUE FOR ROLLY

C     TOLER(38, ) ENDING SURFACE NUMBER FOR ROLLY

C     TOLER(39, ) ALTERNATE SURFACE NUMBER FOR ROLLX

C

C

C                  ADDED 4/27/92 (LBL AND PLBL)

C     80 CHARACTER LABEL (LBL) LINES FOR EACH LENS ARE ADDED WITH

C     THE ARRAYS LBL(0:MAXSUR)*80 AND PLBL(0:MAXSUR)*80 [PERM LENS]

C     THEY ARE USED TO HOLD IDENTIFIER OF OTHER IMPORTANT NOTATIONS

C     REGARDING EACH LENS SURFACE. SEE ALENS(44,SURF) FOR THE FLAG DENOTING

C     OR ABSENCE OF THE NOTATION IN THE NOTATION ARRAYS.

C

C LI AND LIC(FOUR LINES WORTH) ARE STORED IN THE CHARACTER ARRAYS

C LI AND LIC(1 TO 4)

C

C

C        ARE USED TO DEIFINE THE SURFACE.

C

C A LENS FILE MAY HAVE ANY SURFACE A UNIQUE TYPE 1 TO TYPE 50

C IF SURFACE 45 IS A TYPE 30 THEN:

C

C                ALENS(34,45)=30.0

C

C GLASS NAME DATA IS STORED IN GLANAM(0 TO 199,1 TO 2)

C VALID PRIMARY GLASS NAMES ARE: AIR,GLASS

C

C SECONDARY GLASS NAMES ARE TO BE USED AS IDENTIFIERS OR SPECIAL

C QUALIFIERS.

C

C       SOLVES

C

C       SOLVE VALUES AND SPECIFIC TYPES ARE STORED IN THE ARRAY

C       (SOLVE) DIMMENSIONED 0:9,0:199

C       THE SECOND DIMENSION IS THE SURFACE NUMBER TO WHICH THE

C       SOLVE IS ASSIGNED. THE FIRST DIMMENSION IS:

C

C                       6 STORES THE THICKNESS SOLVE TYPE

C

C                               1.0=PY

C                               2.0=PCY

C                               3.0=CAY

C

C                       4 STORES THE THICKNESS SOLVE TYPE

C

C                               4.0=PX

C                               5.0=PCX

C                               6.0=CAX

C

C                       7 STORES THE NUMERIC VALUE ASSOCIATED

C               WITH THE (YZ) PLANE THICKNESS SOLVE (DEFAULTS ARE

C                               ALWAYS ZERO)

C                       3 STORES THE NUMERIC VALUE ASSOCIATED

C               WITH THE (XZ) PLANE THICKNESS SOLVE (DEFAULTS ARE

C                               ALWAYS ZERO)

C                       8 STORES THE CURVATURE SOLVE TYPE (YZ) PLANE

C

C                               1.0=APY

C                               2.0=PIY

C                               3.0=PUY

C                               4.0=APCY

C                               5.0=PICY

C                               6.0=PUCY

C                               7.0=COCY

C

C                       2 STORES THE CURVATURE SOLVE TYPE (XZ) PLANE

C

C                               8.0=APX

C                               9.0=PIX

C                               10.0=PUX

C                               11.0=APCX

C                               12.0=PICX

C                               13.0=PUCX

C                               14.0=COCX

C

C                       9 STORES THE NUMERIC VAULU ASSOCIATED WITH

C       THE (YZ) PLANE CURVATURE SOLVE (DEFAULTS ARE ALWAYS ZER0)

C                       1 STORES THE NUMERIC VAULU ASSOCIATED WITH

C        THE (XZ) PLANE CURVATURE SOLVE (DEFAULTS ARE ALWAYS ZER0)

C

C                       THE ZERO POSITION IN THE FIRST DIMENSION

C                       IS NOT USED.

C

C       THE ARRAY SOLVE IS INITIALIZED IN THE SUBROUTINE LENNS

C

C

C       PIKUPS ARE TRACKED IN ALENS(32,SURF). THE INTEGER VALUE

C       OF THE NUMBER STORED THERE IS THE NUMBER OF PIKUPS STORED

C       ON THAT PARTICULAR SURFACE. THE ACTUAL PIKUP DATA IS

C       STORED IN THE PIKUP ARRAY DIMMENSIONED

C

C               PIKUP(1:6,0:199,1:SPIK) WHERE THE FIRST DIMENSION

C                       STORES

C                       1=PIKUP EXISTENCE FLAG, WHICH IS

C                       1.0 IF THE PIKUP IS PRESENT AND 0.0

C                       IF THE PIKUP IS NOT PRESENT

C                       2 TO 6 STORE THE 5 NUMERIC WORDS

C                       6 STORES THE SOURCE CONFIGURATION FLAG

C                               THE SECOND DIMENSION

C                       STORES DATA FOR EACH OF THE MAXSUR SURFACES

C                               THE THIRD DIMMENSION ALLOWS

C                       FOR UP TO 45 DIFFERENT PIKUPS TO BE

C                       ASSIGNED TO A SURFACE AT ONE TIME

C

C

C       THE THIRD DIMENSION DESIGNATES THE TYPE OF PIKUP

C                       ENCODED AS:

C                       1=RD

C                       2=CV

C                       3=TH

C                       4=CC

C                       5=AD

C                       6=AE

C                       7=AF

C                       8=AG

C                       9=CVTOR

C                       10=RDTOR

C                       11=PRO

C                       12=NPRO

C                       13=YD

C                       14=XD

C                       15=ALPHA

C                       16=BETA

C                       17=GAMMA

C                       18=CLAP

C                       19=COBS

C                       20=GLASS

C                       21=CCTOR

C                       22=ADTOR

C                       23=AETOR

C                       24=AFTOR

C                       25=AGTOR

C                       26=AC

C                       27=AH

C                       28=AI

C                       29=AJ

C                       30=AK

C                       31=AL

C                       32=THOAL

C                       33=ZD

C                       34=PIVX

C                       35=PIVY

C                       36=PIVZ

C                       37=GDX

C                       38=GDX

C                       39=GDX

C                       40=GALPHA

C                       41=GBETA

C                       42=GGAMMA

C                       43=GRT

C                       44=COATING

C     FOR THOAL NW1 IS STARTING SURFACE NUMBER

C     FOR THOAL NW2 IS STOPPING SURFACE NUMBER

C     FOR THOAL NW3 IS MULTIPLICATIVE TERM

C     FOR THOAL NW4 IS ADDITIVE TERM

C     FOR THOAL NW5 IS SOURCE CONFIGURATION FLAG

C*********************************************************************

C       CONFIGURATION DATA IS STORED IN

C               THE ARRAY CONFIG(2:100,1:200)*80

C       WHERE THERE MAY BE 200 80 CHARACTER LINES OF CONFIG DATA

C       FOR ANY ONE OF THE 99 CONFIGS FROM 2 TO 100

C

C       THE VARIABLE CFGCNT(2:100) KEEPS COUNT OF THE NUMBER OF

C       DATA ITEMS IN EACH CONFIGURATION

C

C       CONFIG DATA IS PASSED IN COMMON/CFG/CONFIG,CFGCNT

C

C       FLAGS ASSOCIATED WITH CFGS ARE:

C               F11=1 FOR UPDATE CONFIG(F1=0)

C               F10=1 FOR CONFIG INPUT (F1=0)

C               F12= THE NUBER OF THE CURRENT CONFIGURATION

C               F13 TRACKS U L INSIGE OF CONFIG INPUT OR UPDATE

C               F14 TRACKS U SP INSIDE CONFIG INPUT OR UPDATE

C

C********************************************************************

C

C                       THE PARAXIAL TRACE. (BEGUN 6/22/88)

C

C                       AFTER LENS INPUT OR UPDATE LENS

C

C       THE PARAXIAL TRACE IS BEGUN BY A CALL FROM

C       LNSEOS TO SUBROUTINE PRTRA1 WHENEVER THE COMMAND EOS

C       IS ISSUED FROM THE LENS OR UPDATE LENS LEVEL.

C                       RULES:

C       1. TRACE IS AUTOMATICALLY PERFORMED UPON THE CURRENT

C               LENS WHICH IS ALWAYS CFG 1

C       2. TRACE IS PERFORMED USING THE CURRENTLY DEFINED CONTROL

C               WAVELENGTH WITH THE CURRENTLY DEFINED PRIMARY AND

C               SECONDARY WAVELENGTH PAIRS.

C       3. FIRST PASS THROUGH THE TRACE IGNORES PARAXIAL SOLVES

C               AND SETS THE CORRECT TARGETING VALUES TO

C               CORRECTLY INTERCEPT THE STOP AT THE REQUIRED

C               COORDINATES. THIS TARGET IS REMEMBERED IN SECOND ENTRY

C               OF

C               SCY, SCX, SCY FANG AND SCX FANG

C

C       CASE OF TORIC FLAG OFF

C       PARAXIAL DATA DEVELOPED AND STORED IN THE ARRAY

C       (PXTRAY(M,SURF)) WHERE M RANGES FROM 1 TO 6

C               M:

C                       1=PY(SURF)

C                       2=PUY(SURF)

C                       3=PIY(SURF)

C                       4=PIYP(SURF)

C                       5=PCY(SURF)

C                       6=PUCY(SURF)

C                       7=PICY(SURF)

C                       8=PICYP(SURF)

C               SURF RANGES FROM 0 TO INT(SYSTEM(20))

C               AND SYSTEM(20) CAN REACH A MAX VALUE OF 199

C

C       CASE OF TORIC FLAG ON

C       IN THE YZ PLANE

C       PARAXIAL DATA DEVELOPED AND STORED IN THE ARRAY

C       (PXTRAY(M,SURF)) WHERE M RANGES FROM 1 TO 6

C               M:

C                       1=PY(SURF)

C                       2=PUY(SURF)

C                       3=PIY(SURF)

C                       4=PIYP(SURF) ANGLE OF REFLECTION OR REFRACTION

C                       5=PCY(SURF)

C                       6=PUCY(SURF)

C                       7=PICY(SURF)

C                       8=PICYP(SURF) ANGLE OR REFLECTION OR REFRACTION

C               SURF RANGES FROM 0 TO INT(SYSTEM(20))

C               AND SYSTEM(20) CAN REACH A MAX VALUE OF 199

C       AND IN THE XZ PLANE

C       PARAXIAL DATA DEVELOPED AND STORED IN THE ARRAY

C       (PXTRAX(M,SURF)) WHERE M RANGES FROM 1 TO 8

C               M:

C                       1=PX(SURF)

C                       2=PUX(SURF)

C                       3=PIX(SURF)

C                       4=PIXP(SURF)

C                       5=PCX(SURF)

C                       6=PUCX(SURF)

C                       7=PICX(SURF)

C                       8=PICXP(SURF)

C               SURF RANGES FROM 0 TO INT(SYSTEM(20))

C               AND SYSTEM(20) CAN REACH A MAX VALUE OF 199

C

C       STARTING VALUES: (DITO FOR XZ PLANE TRACE WHEN TORICS PRESENT)

C               PY(SURF=0)=0

C               PY(SURF=1)=SAY

C               PCY(SURF=0)=SCY

C               PCY(SURF=1)=0 ; ALSO SET TO PCYTAR WHICH IS

C       MODIFIED IF ASTOP NOT AT SURF=1

C                       NOW ALSO PUY(0)=SAY/(TH(0))

C       AND                     PUCY(0)=PUCY(1)=SCY/TH(0)

C

C       PARAXIAL EQUATIONS:

C       PU[C]Y'=-CV*P[C]Y*((N'-N)/N')+(N/N')*PU[C]Y

C       (REFRACTION/REFLECTION)

C

C       P[C]Y'=P[C]Y+TH*PU[C]Y'  (TRANSFER)

C

C       PI[C]Y=CV*P[C]Y+PU[C]Y          (INCIDENCE ANGLE)

C

C       PI[C]Y'=(N/N')*PI[C]Y           (REFLECTION/REFRACTION ANGLE)

C

C       REMINDER: SAY,SAX  STORED IN SYSTEM(12) AND SYSTEM(13)

C                 SCY STORED IN SYSTEM(14) AND SYSTEM(15)

C                 SCX STORED IN SYSTEM(16) AND SYSTEM(17)

C                 SCY FANG STORED IN SYSTEM(21) AND SYSTEM(22)

C                 SCX FANG STORED IN SYSTEM(23) AND SYSTEM(24)

C               LENS DATA IN ALENS

C               GLASS DATA IN ALENS AND GLANAM

C

C

C       MACRO STUFF ADDED 3/13/90

C

C       THE MAXIMUM NUMBER OF MACRO LINES IS (MAXLIN) NOT 512

C       MAXLIN IS INITIALIZED IN THE INCLUDE FILE DATMAC.FOR. THIS

C       FILE IS INCLUDED IN ALL MACRO RELATED SUBROUTINES AND INCLUDES

C       ALL APPROPRIATE COMMONS FOR MACRO DATA PASSAGE.

C

C     THE ARRAY DUMMM(1:MAXSUR) VALUES ARE SET TO TRUE OR FALSE

C     DEPENDING ON WHETHER A SURFACE IS A DUMMY OR NOT. tHIS IS

C     DONE IN LNSEOS EACH TIME IT IS CALLED. tHE ARRAY IS NOT WRITTEN

C     TO DISK BUT IS PASSED IN A COMMON BLOCK VIA DATLEN.FOR

C     GLOBAL SURFACE VERTEX ARRAY

C     VERTEX(I,SURF)

C                       I=1  X (GLOBAL X OF ORIGIN)

C                       I=2  Y (GLOBAL Y OF ORIGIN)

C                       I=3  Z (GLOBAL Z OF ORIGIN)

C                       I=4  LX-L DIR COS X-AXIS

C                       I=5  MX-M DIR COS X=AXIS

C                       I=6  NX-N DIR COS X AXIS

C                       I=7  LY (Y-AXIS)

C                       I=8  MY ........

C                       I=9  NY ........

C                       I=10 LZ (Z-AXIS)

C                       I=11 MZ ........

C                       I=12 NZ ........

C

C       THIS PROGRAM CONSISTS OF A CMD OR MAIN LEVEL AND

C       A NUMBER OF SUB- LEVELS.

C

C       FLAG F1=1 REPRESENTS THE CMD LEVEL OF THE PROGRAM

C       FLAG F1=0 REPRESENTS NOT THE CMD LEVEL.

C

C       FLAG F2=1 REPRESENTS NEW MACRO CREATION

C       FLAG F2=0 REPRESENTS NOT NEW MACRO CREATION

C

C       (OTHER FLAG STATUS COMMENTS WILL GO HERE AS THE

C       PROGRAM IS DEVELOPED)

C

C       INITIALIZE THE RAY-DIRECTION COSINE TRACKING FLAGS

C       TO +Z DIRECTION, OTHERWISE THE RAYTRACE GIVES INCORRECT

C       RESULTS.

C

C       START THE PROGRAM ALWAYS AT CMD LEVEL. SET FLAG F1

C

F1=1

C

C       AND FLAG F2 (MACRO CREATION) IS SET TO 0

F2=0

C

C       FLAG F3 IS INITIALLY SET TO 0. WHEN SET TO 1

C       THE MACRO EDIT OR MEDIT LEVEL OF THE PROGRAM

C       IS IN EFFECT. ONLY THE FL COMMAND CAN RESET

C       FLAG F3 TO ZERO.

F3=0

C

C       FLAG F4 WHEN SET MEANS THAT A MACRO IS BEING EXECUTED.

C       THE INITIAL DEFAULT SETTING FOR F4 IS F4 = 0

F4=0

C

C       THE SPECIAL TRACE FLAG TF(NEST) TRACKS STATUS

C       OF TRACE ON OR OFF IN MACRO EXECUTION AT VARIOUS

C       NESTING LEVELS.

C       TF(NEST)=1 IS TRACE ON, TF(NEST)=0 IS TRACE OFF

C       DEFAULT IS TF(NEST)=0

C

TF(0:20)=0

C

C       FLAG F5 TRACKS LENS INPUT MODE WHICH IS INITIALIZED

C       BY THE 'LENS' COMMAND AND TERMINATED WITH EOS (END OF SUBFILE).

C       THE DEFAULT VALUE OF F5 IS 0. DURING LENS INPUT F5=1 AND F1=0.

C

F5=0

C

C       FLAG F6 TRACKS LENS UPDATES INITIALIZED BY THE 'UPDATE'

C       COMMAND WITH THE 'LENS' QUALIFIER WORD. THE DEFAULT VALUE OF

C       F6=0. DURING LENS UPDATING F6=0 AND F1=0.

C

F6=0

C

C       FLAG F7 TRACKS SPSRF (SPECIAL SURFACE) INPUT MODE

C       INITIATED BY THE COMMAND (SPSRF). DEFAULT IS F7=0

C

F7=0

C

C       FLAG F8 TRACKS UPDATE SPSRF (SPECIAL SURFACE)

C       UPDATE MODE BY THE COMMAND (UPDATE SPSRF) DEFAULT VALUE

C       IS F8=0

C

F8=0

C

C       FLAG F9 TRACKS THE SPFIT LEVEL ENTERED BY (SPFIT)

C       DEFAULT VALUE IS F9=0

C

F9=0

C

C       FLAG F10 TRACKS THE CONFIGS INPUT LEVEL ENTERED BY COMMAND

C       (CONFIGS) ITS DEFAULT VALUE IS 0

C

F10=0

C

C       FLAG F11 TRACKS THE CONFIGS UPDATE LEVEL ENTERED BY COMMAND

C       (UPDATE CONFIGS) ITS DEFAULT VALUE IS 0

C

F11=0

C

C

C       FLAG F12 TRACKS THE CURRENT CONFIGURATION NUMBER

C       THE STARTING AND DEFAULT VALUE IS 1

C

F12=1

C

C       FLAGS F13 AND F14 TRACK THE USAGE OF UPDATE LENS

C       AND UPDATE SPSRF FROM WITHIN CONFIGS OR UPDATE

C       CONFIGS AND ARE USED BY QUERRY TO PRINT CORRECT

C       RESPONSE TO A ? AT THE CONFIGS/UPDATE CONFIGS LEVEL

C       DEFAULT VALUES ARE BOTH 0

C

F13=0

F14=0

C

C       FLAG F15 IS SET TO 1 WHEN THE PROCESS OF ACTIVATING

C       A CONFIGURATION IS BEING PERFORMED. OTHERWISE

C       ITS DEFAULT VALUE IS 0

C

F15=0

C

C       FLAG F16 TRACKS WHETHER OR NOT A MACRO OF THE SAME NAME AS

C       AN EXISTING MACRO IS TRYING TO BE CREATED BY THE "MACRO"

C       COMMAND. THE DEFAULT VALUE ID F16=0. IF A MACRO OF THE SAME

C       NAME DOES EXIST, F16 IS SET TO 1

C

F16=0

C

C       FLAG F17 TRACKS THE SPECT OR SPECTRAL ANALYSIS

C       LEVEL OF THE PROGRAM. IF F17=1 THEN F1 IS = 1 NOT 0.

C       SPECT HAS AVAILABLE, MANY CMD LEVEL COMMANDS.

C       AND WE ARE AT THE SPECT SUB-LEVEL OF THE PROGRAM.

C       THE DEFAULT IS F17=0

C

F17=0

C

C       FLAG F18 TRACKS THE TABLE INPUT LEVEL WITHIN THE SPECT

C       LEVEL OF THE PROGRAM. IF F18=1 AND F17=1 THEN

C       AND WE ARE AT THE TABLE SUB-LEVEL OF SPECT.

C       THE DEFAULT IS F18=0

C

F18=0

C

C       FLAG F19 TRACKS THE EXISTENCE OF TABLE DATA.

C       THE DEFAULT IS F19=0 FOR NO DATA.

C

F19=0

C

C       FLAG F20 IS AVAILABLE FOR USE

C

C       FLAG F21 IS USED TO TELL THE PROGRAM WHEN

C       REFRACTIVE INDICES SHOULD BE MADE NEGATIVE.

C       EACH TIME A REFECTIVE SURFACE IS ENCOUNTERED, THE

C       VALUE OF FLAG F21 IS REVERSED. THE DEFAULT VALUE IS

C               F21=0

F21=0

C

C       AT LENS EOS IN SUBROUTINE LNSEOS, ALL RESOLUTION OF

C       SIGN OF REFRACTIVE INDICES IS HANDLED AS ONE OF THE

C       FIRST DUTIES IN THAT SUBROUTINE.

C

C       F22

C       IF F22=0, THE GLASS CATALOG IS NOT REFERENCED. IF

C       F22=1, A CATALOG LOOKUP IS PERFORMED.

C       F22 IS ALWAYS SET TO 1 DURING LENS INPUT. AFTER

C       LENS INPUT OR LENS UPDATE OR CONFIG CHANGE, F22=0

C       THE STARTING DEFAULT IS F22=0

C       DURING LENS UPDATE, THE FOLLOWING OPERATIONS

C       SET F22 TO 1

C                       ANY CHANGE OF MATERIAL

C                       ANY SUFACE DELETION OR INSERTION

C                       ANY CHANGE TO THE REFRACTIVE INDICES

C                       ANY CHANGE TO THE WAVELENGTHS

C       LIB GET ALSO SETS F22=1

C

F22=0

C

C       F23 TRACKS IF TYPE OR SURF WAS ENTERED IN THE

C       SPFIT ROUTINES. IF TYPE IS SET, F23=1,

C       IF SURF IS SET, F23=2

C       DEFAULT IS NOT F23=0

C

F23=0

C

C       F24 TRACKS THE PRESENTS OF DATA TO BE FITTED, THAT IS

C       DATA ACCUMULATED WITH THE PROER COEFFS AND FUNCTIONAL FORM

C       NO DATA IS F24=0, DATA = F24=1

C

F24=0

C

C       F25 IS SET TO 1 IF FITTED DATA EXISTS TO BE EVALUATED.

C       IF A FIT IS NOT PERFORMED TO THE POINT THAT COEFS ARE

C       AVAILABLE FOR EVALUATION, F24=0 (DEFAULT)

C

F25=0

C

C       FLAG F26 IS 1 WHEN A MACRO FUNCTION IS RUNNING

F26=0

C

C       FLAG F27 TRACKS IF THE PROGRAM IS IN THE MERIT LEVEL

C       AND THE UPDATE MERIT (U M) LEVEL

C       IF F27=0, NOT

C       IF F27=1, MERIT CREATION LEVEL

C       IF F27=2, UPDATE MERIT LEVEL

F27=0

C

C       FLAG F28 TRACKS THE PROGRAM OPERATION DURING OPTIMIZATION

C       CALCULATIONS

C       IF F28=0, NOT

C       IF F28=1, OPTIMIZATION CALCULATIONS IN PROGRESS

F28=0

C

C       F35 IS LIKE F28 BUT ONLY USED WITH INTERNAL CALLS TO ITER

C       FLAG F35 TRACKS THE PROGRAM OPERATION DURING OPTIMIZATION

C       CALCULATIONS

C       IF F35=0, NOT

C       IF F35=1, OPTIMIZATION CALCULATIONS IN PROGRESS

F35=0

C

C       FLAG F29 TRACKS THE VARIABLES LEVEL

C       AND THE UPDATE VARIABLES LEVEL

C     IF F29=0 ; NOT

C     IF F29=1 ; VARIABLES

C     IF F29=2 ; UPDATE VARIABLES

F29=0

C

C       FLAG F30 NOT USED

F30=0

C

C       FLAG F31 TRACKS THE PROGRAM OPERATION DURING TOLERANCE

C       CALCULATIONS

C       IF F31=0, NOT

C       IF F31=1, TOLERANCING CALCULATIONS IN PROGRESS

F31=0

C       FLAG F32 TRACKS THE LENSDRAWING INPUT LEVEL

C

C       FLAG 46 IS USED TO TRACK A BLANK MACRO

C

C       FLAG F47 IS USED TO FACILITATE CALLING SUBROUTINE

C       MREA DURING A REPLACEMENT OF A MACRO LINE DURING

C       THE MEDIT PROCESS.

C

C       FLAGS F45,F48 AND F49 ARE USED IN ITF,ILF AND IMF PROCEED

C       PROCEEDURES. THE MAKE ITF,IMF AND ILF WORK CORRECTLY.

C       INITIAL VALUES ARE ZERO. FLAG F33 IS USED FOR IPF (PLOT FILE

C       INITIALIZATION)

F33=0

F45=0

F48=0

F49=0

C

C     F34=1 IF VIE PLOTTING IS BEING DONE

F34=0

C

C       FLAG F50 TRACKS STATUS TO DETERMINE IF A MESSAGE

C       RESPONSE IS REQUIRED FOR BLANK LINE INPUT.

C       DEFAULT IF F50=0

F50=0

C

C     FLAG F51 TRACKS TOLERANCE VARIABLE INPUT AND UPDATE (0,1 OR 2)

C     FLAG F52 TRACKS TOLERANCE COMPENSATOR INPUT AND UPDATE (0,1 OR 2)

C     FLAG F53 TRACKS TOLERANCE OPERAND INPUT AND UPDATE (0,1 OR 2)

C     FLAG F54 TRACKS TOLERANCE FOCRIT INPUT AND UPDATE (0,1 OR 2)

C     FLAG F55 TRACKS OPERATION OF THE TOLERANCE ANALYSIS

C       F55=0 MEANS NONE BEING DONE

C       F55=1 MEANS SENSI

C       F55=2 MEANS INVERSE SENSI

C       F55=3 MEANS MONTE-CARLO

F51=0

F52=0

F53=0

F54=0

F55=0

C

C     FLAGS 57 TO 100 ADDED ON 9/5/92 FOR FUTURE EXPANSION AND SET TO

C     ZERO HERE

C     F56 IS 0 IF DEFORMABLE SURFACE DATA HAS NOT BEEN READ FROM DISK

C     AND ID 1 IF IT ALREADY HAS BEEN

F56=0

C

C

C     FLAG F57 IS 0 IF NO AUTOFUNC FUNCTION IS TO BE EXECUTED, IT IS 1

C     IF THERE IS AN UNRESOLVED AUTOFUNC FUNCTION

F57=0

C

C

C

C

C     FLAG F58 IS 1 IF FANS ARE BEING GENERATED, ELSE F58=0

C

F58=0

C

C

F59=0

F60=0

F61=0

F62=0

F63=0

F64=0

F65=0

F66=0

F67=0

F68=0

F69=0

F70=0

F71=0

F72=0

F73=0

F74=0

F75=0

F76=0

F77=0

F78=0

F79=0

F80=0

F81=0

F82=0

F83=0

F84=0

F85=0

F86=0

F87=0

F88=0

F89=0

F90=0

F91=0

F92=0

F93=0

F94=0

F95=0

F96=0

F97=0

F98=0

F99=0

F100=0

C

C

C       PATCH CODE TO READ HEXAGON CONFIGS RECORDS IS IN UTILITY2.FOR IN THE PRO3 SUBROUTINE, JUST AHEAD OF

C       THE CALL TO CONTRO

LENO EXCEL FLAG SYSTEM This is used to track if a column is not needed in the config being output.

LENOEX(I) is a logical array. .TRUE. means include the column, .false. means exclude it.

I DESIGNMATES

1  = RD

2  = TH

3  = MATERIAL

4  = CC

5  = AC

6  = AD

7  = AE

8  = AF

9  = AG

10 = AH

11 = AI

12 = AJ

13 = AK

14 = AL

15 = XD

16 = YD

17 = ZD

18 = TILT TYPE OR DEC ONLY

19 = ALPHA

20 = BETA

21 = GAMMA

22 = INDEX AT WV1

23 = INDEX AT WV2

24 = INDEX AT WV3

25 = INDEX AT WV4

26 = INDEX AT WV5

27 = INDEX AT WV6

28 = INDEX AT WV7

29 = INDEX AT WV8

30 = INDEX AT WV9

31 = INDEX AT WV10

32 = GLOBAL XD

33 = GLOBAL YD

34 = GLOBAL ZD

35 = GLOBAL ALPHA

36 = GLOBAL BETA

37 = GLOBAL GAMMA

38 = SPECIAL SURFACE TYPE

39 TO 134 = SPECIAL SURFACE COEFFICIENTS

135= SOLVE TYPE

136= SOLVE DATUM

137= PIKUP TYPE

138= PIKUP NW1

139= PIKUP NW2

140= PIKUP NW3

141= PIKUP NW4

142= PIKUP NW5

C       VARIABLE SUBFILE

C       VARABL(I,J) WHERE I COUNTS THE NUMBER OF VARIABLE ENTRIES

C       AND J TAKES ON THE FOLLOWING VALUES AND MEANIINGS.

C

C       J=1  > 1 THROUGH 150 AND 250 AND ON FOR ACT., A VARIABLE TYPE DESIGNATOR

C       J=2  > 1 THROUGH MAXCFG, THE CONFIGURATION DESIGNATOR

C       J=3  > 0 THROUGH MAXSUR, THE SURFACE NUMBER DESIGNATOR

C       J=4  > VARIABLE CURRENT VALUE

C       J=5  > VARIABLE PREVIOUS VALUE

C       J=6  > LAST VARIABLE CHANGE VALUE

C       J=7  > WT, THE WEIGHTING FACTOR

C       J=8  > DINCR (DERIVATIVE CHANGE VALUE)

C       J=9  > LOW LIMIT VALUE

C       J=10 > HIGH LIMIT VALUE

C       J=11 > SEQUENTIAL IDENTIFIER IN THE AUXCFG FILES

C       J=12 > DEFAULT DINCR FLAG (1=DEFFAULT, 0=USER SET)

C       J=13 > ORIGINAL VALUE

C       J=14 > ENTRY NUMBER IN THE CFADD ARRAY THAT REFERS TO

C              THIS VARIABLE

C       J=15 > DF2

C       J=16 > DF4

C       J=17 > DF5

C

C       THE ARRAY OPERND AND OPNAM STORE MERIT INFORMATION

C       IT IS PASSED IN COMMON IN THE INCLUDE FILE DATSUB.FOR

C

C       OPERND(I,J) WHERE I COUNTS THE NUMBER OF OPERAND ENTRIES

C       AND J TAKES ON THE FOLLOWING VALUES AND MEANIINGS.

C

C       J=1  > 1 THROUGH 10, THE NUMBER OF THE FUNCTION IN WHICH

C               THE ACTUAL OPERAND IS CALCULATED (COMMAND WORD)

C         (0 IS PREDEFINED OPERAND)

C

C       J=2  > TARGET VALUE OF THE OPERAND (NUMERIC WORD #1)

C       J=3  > OPERAND ORIGINAL VALUE

C       J=4  > OPERAND CURRENT VALUE

C       J=5  > OPERAND PREVIOUS VALUE

C       J=6  > LAST OPERAND CHANGE VALUE (CURRENT-PREVIOUS)

C       J=7  > WT, THE WEIGHTING FACTOR DURING OPTIMIZATION (NUMERIC WORD #2)

C

C       J=8  > NUMBER OF THE GENERAL PURPOSE REGISTER CONTAINING

C               THE ACTUAL OPERAND VALUE AS RETURNED FROM THE

C               CALCULATING FUNCTION (NUMERIC WORD #3) (VALUES 1 TO 300)

C               (A DEFAULT ENTRY HERE CAUSES THE ACCUMULATOR TO BE USED)

C       FOR PREDEFINED OPERAND, IT IS THE (I) INPUT VALUE

C

C       J=9  > OPTIONAL NW1 FOR THE FUNCTION (USING NSUB) (NUMERIC WORD #4)

C       J=10 > OPTIONAL NW2 FOR THE FUNCTION (USING NSUB) (NUMERIC WORD #5)

C     FOR PREDEFINED OPERANDS J=9 AND J=10 ARE THE (J) AND (K) INPUT VALUES

C       J=11 > DEFAULT FLAG FOR NW1

C       J=12 > DEFAULT FLAG FOR NW2

C       J=13 > COR MODE 1=COR, 0=BYP, BLO,GTE=-2, BHI,LTE=+2, HLD=10

C       J=14 > SQUARE ROOT OF WEIGHT TIMES (CURRENT VAL-TARGET))

C       J=15 > 0 BEFORE THE FIRST CALCULATION OF OPERAND VALUES

C              1 AFTER THE FIRST CALULATION

C       J=16 > CFG # FOR PREDEFINED OPERANDS (NW3)

C       J=17 > CODE FOR THE PREDEFINED OPERANDS

C       J=18 > DEFAULT FLAG FOR W3 = DBLE(DF3)

C       J=19 > 0 IF OP CALCULABLE, 1 IF NOT

C       J=20 > RESERVED FOR EXPANSION

C             1=X

C             2=Y

C             3=Z

C             4=L

C             5=M

C             6=N

C             7=DX

C             8=DY

C             9=DR

C            10=DXA

C            11=DYA

C            12=DRA

C            13=XANG

C            14=YANG

C            15=OPL

C            16=OPD

C            17=OPDW

C            18=LOLD

C            19=MOLD

C            20=NOLD

C            21=LEN

C            22=AII

C            23=AIP

C            24=LN

C            25=MN

C            26=NN

C            27=PXPX

C            28=PXPY

C            29=PYPX

C            30=PYPY

C            31=PXAPX

C            32=PXAPY

C            33=PYAPX

C            34=PYAPY

C            35=DXDX

C            36=DXDY

C            37=DYDX

C            38=DYDY

C            39=DXADX

C            40=DXADY

C            41=DYADX

C            42=DYADY

C            43=XREF

C            44=YREF

C            45=ZREF

C            46=LREF

C            47=MREF

C            48=NREF

C            49=LREFOL

C            50=MREFOL

C            51=NREFOL

C            52=IREF

C            53=IPREF

C            54=XAREF

C            55=YAREF

C            56=LNREF

C            57=MNREF

C            58=NNREF

C            59=GLX

C            60=GLY

C            61=GLZ

C            62=GLL

C            63=GLM

C            64=GLN

C            65=GLLOLD

C            66=GLMOLD

C            67=GLNOLD

C            68=LENREF

C            69=OPLREF

C            70=RD

C            71=CV

C            72=TH

C            73=CC

C            74=AC

C            75=AD

C            76=AE

C            77=AF

C            78=AG

C            79=RDTOR

C            80=CVTOR

C            81=CCTOR

C            82=ADTOR

C            83=AETOR

C            84=AFTOR

C            85=AGTOR

C            86=ALPHA

C            87=BETA

C            88=GAMMA

C            89=VNUM

C            90=PARTL

C            91=INDEX

C            92=XD

C            93=YD

C            94=XVERT

C            95=YVERT

C            96=ZVERT

C            97=LXVERT

C            98=MXVERT

C            99=NXVERT

C           100=LYVERT

C           101=MYVERT

C           102=NYVERT

C           103=LZVERT

C           104=MZVERT

C           105=NZVERT

C           106=LENGTH

C           106=OAL

C           107=MLENGTH

C           107=OPTLEN

C           108=ET

C           108=ETY

C           109=ETX

C           110=SHAPEFAC

C           111 TO 206 = C1 THROUGH C96

C           207=PWRY

C           208=PWRX

C           209=FLCLTHX

C           210=FLCLTH OR FLCLTHY

C           211=PY

C           212=PX

C           213=PCY

C           214=PCX

C           215=PUY

C           216=PUX

C           217=PUCY

C           218=PUCX

C           219=PIY

C           220=PIX

C           221=PICY

C           222=PICX

C           223=PIYP

C           224=PIXP

C           225=PICYP

C           226=PICXP

C           227=PACY

C           228=PACX

C           229=PLCY

C           230=PLCX

C           231=SACY

C           232=SACX

C           233=SLCY

C           234=SLCX

C           235=IMDISX

C           236=IMDISY

C           237=CENTX

C           238=CENTY

C           239=RMSX

C           240=RMSY

C           241=RMS

C           242=RSSX

C           243=RSSY

C           244=RSS

C           245=RMSOPD

C           246=ZERN37

C           247=MAGX

C           248=MAGY

C           249=MAGXOR

C           250=MAGYOR

C           251=FFLX

C           252=FFLY

C           253=BFLX

C           254=BFLY

C           255=FFNX

C           256=FFNY

C           257=BFNX

C           258=BFNY

C           259=EFLX

C           260=EFLY

C           261=ENDIAX

C           262=ENDIAY

C           263=EXDIAX

C           264=EXDIAY

C           265=ENPOSX

C           266=ENPOSY

C           267=ENPOSZ

C           268=EXPOSX

C           269=EXPOSY

C           270=EXPOSZ

C           271=FNUMX

C           272=FNUMY

C           273=OBFNUMX

C           274=OBFNUMY

C           275=ENPDIAX

C           276=ENPDIAY

C           277=EXPDIAX

C           278=EXPDIAY

C           279=PUPDIAX

C           280=PUPDIAY

C           281=PUPDISX

C           282=PUPDISY

C           283=CHFIMX

C           284=CHFIMY

C           285=GPX

C           286=GPY

C           287=GPUX

C           288=GPUY

C           289=GPCX

C           290=GPCY

C           291=GPUCX

C           292=GPUCY

C           293=DIST

C           294=XFOC

C           295=YFOC

C           296=AST

C           297=SA3

C           298=XSA3

C           299=CMA3

C           300=XCMA3

C           301=AST3

C           302=XAST3

C           303=DIS3

C           304=XDIS3

C           305=PTZ3

C           306=XPTZ3

C           307=SA5

C           308=XSA5

C           309=CMA5

C           310=XCMA5

C           311=AST5

C           312=XAST5

C           313=DIS5

C           314=XDIS5

C           315=PTZ5

C           316=XPTZ5

C           317=TOBSA

C           318=XTOBSA

C           319=SOBSA

C           320=XSOBSA

C           321=ELCMA

C           322=XELCMA

C           323=TAS

C           324=XTAS

C           325=SAS

C           326=XSAS

C           327=SA7

C           328=XSA7

C           329=SA3P

C           330=XSA3P

C           331=CMA3P

C           332=XCMA3P

C           333=AST3P

C           334=XAST3P

C           335=DIS3P

C           336=XDIS3P

C           337=PTZ3P

C           338=XPTZ3P

C           339=SA5P

C           340=XSA5P

C           341=CMA3P

C           342=XCMA3P

C           343=AST5P

C           344=XAST5P

C           345=DIS5P

C           346=XDIS5P

C           347=PTZ5P

C           348=XPTZ5P

C           349=TOBSAP

C           350=XTOBSAP

C           351=SOBSAP

C           352=XSOBSAP

C           353=ELCMAP

C           354=XELCMAP

C           355=TASP

C           356=XTASP

C           357=SASP

C           358=XSASP

C           359=SA7P

C           360=XSA7P

C           361=SA3S

C           362=XSA3S

C           363=CMA3S

C           364=XCMA3S

C           365=AST3S

C           366=XAST3S

C           367=DIS3S

C           368=XDIS3S

C           369=PTZ3S

C           370=XPTZ3S

C           371=SA5S

C           372=XSA5S

C           373=CMA5S

C           374=XCMA5S

C           375=AST5S

C           376=XAST5S

C           377=DIS5S

C           378=XDIS5S

C           379=PTZ5S

C           380=XPTZ5S

C           381=TOBSAS

C           382=XTOBSAS

C           383=SOBSAS

C           384=XSOBSAS

C           385=ELCMAS

C           386=XELCMAS

C           387=TASS

C           388=XTASS

C           389=SASS

C           390=XSASS

C           391=SA7S

C           392=XSA7S

C           393=SA5I

C           394=XSA5I

C           395=CMA5I

C           396=XCMA5I

C           397=AST5I

C           398=XAST5I

C           399=DIS5I

C           400=XDIS5I

C           401=PTZ5I

C           402=XPTZ5I

C           403=TOBSAI

C           404=XTOBSAI

C           405=SOBSAI

C           406=XSOBSAI

C           407=ELCMAI

C           408=XELCMAI

C           409=TASI

C           410=XTASI

C           411=SASI

C           412=XSASI

C           413=SA7I

C           414=XSA7I

C           415=PSA3

C           416=XPSA3

C           417=PCMA3

C           418=XPCMA3

C           419=PAST3

C           420=XPAST3

C           421=PDIS3

C           422=XPDIS3

C           423=PPTZ3

C           424=XPPTZ3

C           425=PSA3P

C           426=XPSA3P

C           427=PCMA3P

C           428=XPCMA3P

C           429=PAST3P

C           430=XPAST3P

C           431=PDIS3P

C           432=XPDIS3P

C           433=PPTZ3P

C           434=XPPTZ3P

C           435=PSA3S

C           436=XPSA3S

C           437=PCMA3S

C           438=XPCMA3S

C           439=PAST3S

C           440=XPAST3S

C           441=PDIS3S

C           442=XPDIS3S

C           443=PPTZ3S

C           444=XPPTZ3S

C           445=PTZCV

C           446=XPTZCV

C           447=AH

C           448=AI

C           449=AJ

C           450=AK

C           451=AL

C           452='GBRADX'

C           453='GBRADY'

C           454='GBDISX'

C           455='GBDISY'

C           456='GBRCVX'

C           457='GBRCVY'

C           458='GBWAISTX'

C           459='GBWAISTY'

C           460='MGOTF'

C           461='PGOTF'

C           462='MDOTF'

C           463='PDOTF'

C           460='GOTFM'

C           461='GOTFP'

C           462='DOTFM'

C           463='DOTFP'

C           464='RED'

C           465='REDCEN'

C           466='FISHDIST'

C           467='ZD'

C           468='SYMX'

C           469='SYMY'

C           470='ASYMX'

C           471='ASYMY'

C           472='PACM'

C           473='PACZ'

C           474='SACM'

C           475='SACZ'

C           476='PLCM'

C           477='PLCZ'

C           478='SLCM'

C           479='SLCZ'

C           480='CTSX'

C           481='CTSY'

C           482='SCEX'

C           483='SCEY'

C           484='GREYS'

C           485='PIVX'

C           486='PIVY'

C           487='PIVZ'

C           488='N1'

C           489='N2'

C           490='N3'

C           491='N4'

C           492='N5'

C           493='N6'

C           494='N7'

C           495='N8'

C           496='N9'

C           497='N10'

C           498='ABBE'

C           499='DPART'

C           500='CLPX'

C           501='CLPY'

C           502='GDX'

C           503='GDY'

C           504='GDZ'

C           505='GALPHA'

C           506='GBETA'

C           507='GGAMMA'

C           508='GRS'

C           509='WEIGHT'

C           510='DMINUSD'

C           511='COST'

C           512='MACOPT'

C           513='RMSYX'

C           514='CLEARX'

C           515='CLEARY'

C           801='ACT'